CITY OF HUNTINGBURG

Application for Excavation Permit









Permit No._______________








      Date Received_______________

Address of Excavation_____________________________________________________________

Reason for Excavation (circle ones appropriate):  Sewer    Water    Gas    Phone    Storm Drain    Other___________________________________________________________

Public Property to be Disturbed (circle ones applicable)  Street    Alley    Sidewalk    Curb    Paved    Gravel    Grass    Other________________________________________

Estimated Start Date___________________Estimated Completion Date______________

Owner Name__________________________

Address_________________________________________Phone___________________

Contractor Name__________________________________

Address_________________________________________Phone___________________

Names and after-hours phone numbers of Contractor’s contact persons:

Name and Phone No.



Name and Phone No.

DRAWING OF EXCAVATION

CITY OF HUNTINGBURG

Standards for Excavating In a Public / Right-of-Way

The purpose of this policy is to establish quality standards for excavations within City right-of-way or roadways in the City of Huntingburg.

1. Permit Fee - $25.  All fees will be paid prior to issuance of the permit or on the next business day.

2. Permit – Prior to any work in a public road and / or public right-of-way, an application for a permit to cut or bore into a City street and / or City right-of-way must be completed and approved.

3. Notification – Prior to any work being performed, the permittee will notify the Indiana Underground Plant Protection Service (Holey Moley) at 800-382-5544 and the City’s Office Manager at 812-683-2211 at least 2 working days prior to the scheduled start date and time, the expected duration of work, and whether any streets or sidewalks will be partially or completely closed to traffic.  If this work is delayed, this information will need to be relayed again to the Office Manager.

4. Where any public street or sidewalk is partially or completely closed to pedestrian or auto traffic, traffic shall be prevented from entering the site or will be routed through the site by barricades and lights furnished by the contractor, and as specified and required by the Street Superintendent.

5. Work will be to the satisfaction of the Huntingburg Street Superintendent or his agents.

6. The work in this permit will be conducted only during daylight hours.

7. Cutting – Prior to any excavation work, all hard surfaced roads (asphalt, chip/seal or concrete) will be saw cut to a minimum depth of 4”.

8. Excavation – All excess excavated material will be removed from right-of-way.  Excavation will meet all required safety standards and will be barricades to protect the public.  The contractor is responsible for all traffic control devices and restoring all sidewalks and curbs to the standards of the City of Huntingburg.

9. Backfill – Utility trenches in the roadway and 2 feet beyond will be backfilled with “flowable fill.”  Flowable fill will be for utility trench backfill and achieve a minimum compressive strength of 75 psi and a maximum compressive strength of 250 psi at 14 days as tested with a standard penetrometer (ASTM C-403).  All products must be tested and approved by the City of Huntingburg prior to its use in a roadway or right-of-way.

Approved mix designs of flowable fill are as follows:

a. Central Concrete Supply, LLC – “Rapid Set”;, or

b. IMI – “Quickset – 1204”.

Alternate mix designs will require approval of the City of Huntingburg.  Flowable fill will remain diggable with a standard backhoe.  Trenches will be filled with flowable fill to within 4 inches from the surface.  Material will be protected from traffic until adequate strength has been achieved.  Backfill outside of roadway will be with excavated material and compacted lifts.

10. Resurfacing – Utility trenches will be resurfaced to match existing road surface.  Asphalt and chip/seal roads will be resurfaced with 3” of HAC base and 1” of HAC surface or capped with 4” of 4000 psi strength concrete.

11. Duration of Repair – Utility trenches will be limited to 5 working days from the start of the excavation work until the completion of the resurfacing and opening of the road to traffic.  If the work is delayed, the Street Superintendent or his agent will be notified as to when work is scheduled for completion.

12. Proof of insurance per the following schedule is attached or on file with the City’s Safety Director:

Coverage






Limits

General Liability


Aggregate BI & PD





2,000,000


Per Occurrence BI & PD




1,000,000

Auto, owned or non-Owned – Per Occurrence


Liability BI & PD





1,000,000


Uninsured/Underinsured Motorist



     50,000

Workers’ Compensation


Proof of Statutory Compliance

Manufacturer’s and Contractor’s Liability


Aggregate





1,000,000


Per Occurrence





1,000,000


Property Damage





   100,000

Builder’s Risk – 100% of Completed Value


Minimum Deductible



                         100

I certify that this application is true and correct, and I agree to indemnify and hold harmless the City of Huntingburg and its employees for any loss, liability or damage that may result or accrue from or because of the applicant’s negligence in performing the work under this permit, or providing the necessary barriers and warning devices, or disregarding any legitimate order of the Street Superintendent or his agents relating to any excavation or tunnel.

_____________________________________________________________


____________________________

Signature of Owner or Owner’s Representative





Date

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

OFFICE USE

Permit Fee:  $__________
Date Permit Issued:  __________
Approved by:  _______________________

Route copies to:  Office Manager, Utilities, Police, Safety, Street, Wastewater
